Khepera

Tennis Ball Push

Judy Kennedy

MCS5503

Fall 2002

Designed, manufactured and distributed by K-Team S.A., Switzerland.

Introduction

The intention of this project is to push a tennis ball towards a goal using the Khepera robot. The goal is to be a dark wall. This task involved several steps. Each will be explained below.

Step 1: Download the software.

Two files were downloaded from http://k-team.com. Kros-i686-pc-cygwin-2001022.tar.gz contains many sample files and header files necessary to use the Khepera in a Windows application. KTProject.exe is an executable that helps manage projects and communicate with the Khepera. All documentation for the Khepera is also available at this site, including valuable information on KOS, the Khepera operating system.

Step 2: Verify the equipment.

I received a Khepera unit with a K213 turret attachment. The K213 is a camera that takes a linear 62 pixel wide 8 bit image. I ran the Braitenburg program and the troubleshooting program on the Khepera with success. I then tried to download a simple test program into the Khepera. I could not achieve a successful download. After sending an email to the makers of Khepera, I was able to complete the download.

Step 3: Verify communication.

I spent a large amount of time trying to establish complete communication with the Khepera. The following steps will help the next person with this task.

- When a project is created in KTProject, select 38400 baud.

- Edit the make file in the project. Make sure it specifies 38400 baud.

- To download a file into the Khepera, you should be in mode 6. The build command line should be “gmake <filename>.dwn”.

Step 4: Create and test the program.

The program I wrote is included at the end of this report.

Step 5: Download the program to the EEPROM and run.

I was not able to achieve a successful download of the program to the EEPROM. However, I can still run the program with the cable attached.

The Program

The program has three main tasks. Since the KOS allows for multitasking, all three tasks run simultaneously.

Task 1: It changes the state of an LED every ½ second. It is a visual way to tell that the program is running.

Task 2: It finds the home (goal) location and points the Khepera towards it. I used input from the K213 to determine Khepera’s position in reference to the goal. It then pivots either left or right to get back on track.

Task 3: It looks for the tennis ball and pushes it. I used the 8 proximity sensors for this input. The Khepera turns left of right based on the input of the sensors. It does a turn instead of a pivot so it can cover more surface area if the ball gets away.

Results

The program works, but not completely. I did notice several points that would need to be improved if there was more time.

- The K213 units for each Khepera output very different numbers. The value of the goal is hard coded into the program. This may require some tweaking at the demonstration. Light also plays a great role here. I would want to include a learning function here where the first thing the Khepera does is learn its surroundings.

- This program does not work well on uneven surfaces. I used a hardwood floor for my testing and found that the grain in the wood affected my test. Unleveled surfaces would also have disastrous results.

- Task 2 needs to be improved. I found that I could only use one movement at a time. For example, I was not able to back up, rotate, move forward, then rotate again as I wanted to. The only action run, was the last action called out. This area needs further debugging.